Non-Normal Situation Guidelines:

- Recognize the Non-Normal:

    - Call out indications and lights.

- Maintain Airplane Control:

    - Establish an acceptable flight path. FLY THE PLANE.

- Analyse and Confirm the Situation:

    - Analyse and state the facts. Review all caution lights and warnings to positively identify the malfunction(s)

- Memory Items:

    - At direction of the PF perform all applicable memory items.

- Short term plan:

      - Quick recap

      - Control Flight path. (Holding/Vectors/HDG, Altitude & speed)

      - Inform ATC (MAYDAY, PANPAN).

      - In case of immediate danger: do not delay and consider an immediate landing.

      - (Time permitting: Inform Pax in case of obvious malfunction.)

- Non-Normal Checklist:

    - With the Airplane not in a critical phase of flight PF calls for the NNC.

- Evaluate the Need to Land:

    - Apply the “Plan to land at the nearest suitable airport.” criteria, or

    - Determine if continuation compromises safety, or

    - Other reason for an early landing are applicable (DDP, company requirements, etc..)

- Follow-Up:

    - Recap:

      - What happened?

      - What have we done?

      - What is the result?

    - Asses Airplane status.

      - Perform a panel scan,

      - Press recall,

      - Check circuit breakers,

      - Other NNC’s.

    - Asses Fuel status. (Fuel in airtime left)

    - NOTAMS, Weather and Performance.

    - Make your Plan: Hold, Continue, divert, return.

    - Inform:

      - ATC

      - Cabin crew

      - Passengers

      - Company

RMAMSNEF

RAF NoWePI